Московский авиационный институт

(национальный исследовательский университет)

Институт № 8 «Информационные технологии и прикладная математика»

Лабораторная работа №3

по курсу «Теоретическая механика»

Уравнение Лагранжа

Выполнил студент группы М8О-207Б-20

Алапанова Эльза Халилевна

Преподаватель: Чекина Евгения Алексеевна

Оценка:

Дата:                  20/12/2021

Москва, 2021

Вариант №«5»

Задание:

Написать на языку python программу визуализирующую кинематику плоского движения механической системы или сложного движения, согласно варианту, используя свободные координаты полученные из уравнения Лагранжа. Кроме анимации системы вывести справа в том же окне графики скоростей обозначенных точек системы.

Механическая система:

Текст программы

Основная :

import numpy as np
import math
import matplotlib.pyplot as plt
from matplotlib.animation import FuncAnimation
import scipy.integrate
from scipy.integrate import odeint
import sympy as sp

def formY(y, t, fV, fOm):
    y1,y2,y3,y4 = y
    dydt = [y3,y4,fV(y1,y2,y3,y4),fOm(y1,y2,y3,y4)]
    return dydt

#defining parameters
m = 2
R = 2
phi0 = sp.pi / 2
ksi0 = 0.1
g = 9.8

#defining t as a symbol (it will be the independent variable)
t = sp.Symbol('t')

#defining ksi, phi, V=dksi/dt and om=dphi/dt as functions of 't'
ksi = sp.Function('ksi')(t)
phi = sp.Function('phi')(t)
V = sp.Function('V')(t)
om = sp.Function('om')(t)

#constructing the Lagrange equations
#1 defining the kinetic energy
# v2 = om**2 * (R**2 + ksi**2) + V**2 - 2 * V * om * R
# w = V / R - om
# J = 1 / 2 * m * R**2
# TT = (m*v2)/2 + (J*w**2)/2
TT = 0.5*m*(om**2 * (R**2+ksi**2) + V**2 - (2 * om * V * R)) + 0.25 * m * R**2 * (V / R - om)**2
#2 defining potential energy
Pi = -m*g*(ksi*sp.cos(phi)-R*sp.sin(phi))
#Lagrange function
L = TT-Pi

#equations
ur1 = sp.diff(sp.diff(L,V),t)-sp.diff(L,ksi)
ur2 = sp.diff(sp.diff(L,om),t)-sp.diff(L,phi)
print(ur1)
#isolating second derivatives(dV/dt and dom/dt) using Kramer's method
a11 = ur1.coeff(sp.diff(V,t),1)
a12 = ur1.coeff(sp.diff(om,t),1)
a21 = ur2.coeff(sp.diff(V,t),1)
a22 = ur2.coeff(sp.diff(om,t),1)
b1 = -(ur1.coeff(sp.diff(V,t),0)).coeff(sp.diff(om,t),0).subs([(sp.diff(ksi,t),V), (sp.diff(phi,t), om)])
b2 = -(ur2.coeff(sp.diff(V,t),0)).coeff(sp.diff(om,t),0).subs([(sp.diff(ksi,t),V), (sp.diff(phi,t), om)])

detA = a11*a22-a12*a21
detA1 = b1*a22-b2*a21
detA2 = a11*b2-b1*a21

dVdt = detA1/detA
domdt = detA2/detA

countOfFrames = 50

# Constructing the system of differential equations
T = np.linspace(0, 12, countOfFrames)
# Pay attention here, the function lambdify translate function from the sympy to numpy and then form arrays much more
# faster then we did using subs in previous lessons!
fV = sp.lambdify([ksi,phi,V,om], dVdt, "numpy")
fOm = sp.lambdify([ksi,phi,V,om], domdt, "numpy")
y0 = [5, -1, -10, 0]
sol = odeint(formY, y0, T, args = (fV, fOm))

#sol - our solution
#sol[:,0] - ksi
#sol[:,1] - phi
#sol[:,2] - dksi/dt
#sol[:,3] - dphi/dt

Ksi = sol[:,0]
Phi = sol[:,1]
print(sol[:,1])
print(sol[:,0])
Steps = 50
t = np.linspace(0, 10, Steps)

X_O = 3 # координаты точки О
Y_O = 10

X_A = Ksi * np.sin(Phi) + X_O
Y_A = - Ksi * np.cos(Phi) + Y_O

X_C = X_O + Ksi * np.sin(Phi) + R * np.cos(Phi)
Y_C = Y_O - Ksi * np.cos(Phi) + R * np.sin(Phi)

angle = np.linspace(0, np.pi*2, 150)
X_Circle = R*np.cos(angle)
Y_Circle = R*np.sin(angle)

X_Ground = [0, 6] # это подвес, на котором держится точка О
Y_Ground = [10, 10]
lSt= 10
fig = plt.figure(figsize=[lSt + 0.5, lSt + 0.5])
ax = fig.add_subplot(1, 2, 1)
ax.axis('equal')
ax.set(xlim=[X_O - (lSt + 0.5), X_O + (lSt + 0.5)],
       ylim=[Y_O/2 - (lSt + 0.5), Y_O/2 + (lSt + 0.5)])

ax.plot(X_Ground, Y_Ground, color='black', linewidth=3)
Drawed_Circle = ax.plot(X_C[0]+X_Circle, Y_C[0]+Y_Circle)[0]
Line_OA = ax.plot([X_O, Y_O], [X_A[0], Y_A[0]])[0] # линия, соединяющая точки O и А

Point_O = ax.plot(X_O, Y_O, marker='o', markersize=10)[0]
Point_A = ax.plot(X_A[0], Y_A[0], marker='o')[0]
Point_C = ax.plot(X_C[0], Y_C[0], marker='o')[0]

ax1 = fig.add_subplot(4, 2, 2)
ax1.plot(T, sol[:,2])
plt.title('dksi/dt')
plt.xlabel('t values')
plt.ylabel('V values')

ax1 = fig.add_subplot(4, 2, 6)
ax1.plot(T, sol[:,3])
plt.title('dphi/dt')
plt.xlabel('t values')
plt.ylabel('Om values')

def Kino(i):
    Point_O.set_data(X_O, Y_O)
    Point_A.set_data(X_A[i], Y_A[i])
    Line_OA.set_data([X_O, X_A[i]], [Y_O, Y_A[i]])

    Point_C.set_data(X_C[i], Y_C[i])
    Drawed_Circle.set_data(X_C[i]+X_Circle, Y_C[i]+Y_Circle)
    return [Point_O, Point_A, Point_C, Line_OA]

anima = FuncAnimation(fig, Kino, frames=Steps, interval=10)

plt.show()

Результат работы: